PROJECT REPORT

Enterprise Network Design with VLANs, Trunking,

Inter-VLAN Routing, DHCP and DNS

Submitted By

SAROJ YADAV

Roll-No: 34

Bachelor's Program in Computer Science

March 2026

# 1. Introduction

This project involves the design and configuration of an enterprise network consisting of two routers and four switches. The network implements Virtual Local Area Networks (VLANs) spanning multiple switches using IEEE 802.1Q trunking. Inter-VLAN routing is achieved using the Router-on-a-Stick method on Router0. A DHCP server provides automatic IP addressing for all end devices, and a DNS server handles internal hostname resolution.

# 2. Objectives

- Design a network using 2 routers (Cisco 1941) and 4 switches (Cisco 2960-24TT)

- Configure VLANs spanning multiple switches using trunk links

- Implement inter-VLAN routing using Router-on-a-Stick on Router0

- Deploy a DHCP server for automatic IP address assignment

- Configure a DNS server for internal hostname resolution

- Verify configuration using show vlan brief and show interfaces trunk commands

# 3. Network Topology

The network follows a hierarchical design with Router0 as the core routing device, Switch1 as the central distribution switch, and Switch0, Switch2, and Switch3 as access switches serving different departments.

Figure 1: Network Topology Diagram

# 4. Device & IP Address Plan

## 4.1 Device Summary

| Device | Model | Role | Interface | IP Address |
| --- | --- | --- | --- | --- |
| Router0 | Cisco 1941 | Edge + Inter-VLAN | Gi0/1 | 10.0.0.1/30 |
| Router1 | Cisco 1941 | Distribution | Gi0/0 | 192.168.2.1/24 |
| Router1 | Cisco 1941 | Distribution | Gi0/1 | 10.0.0.2/30 |
| Switch1 | 2960-24TT | Core L2 Switch | VLAN 99 | 192.168.99.1/24 |
| Switch0 | 2960-24TT | Dept A Access | VLAN 99 | 192.168.99.11/24 |
| Switch2 | 2960-24TT | Dept B/C Dist. | VLAN 99 | 192.168.99.12/24 |
| Switch3 | 2960-24TT | Dept C Access | VLAN 99 | 192.168.99.13/24 |
| Server0 | Server-PT | DHCP + DNS | Fa0 | 192.168.100.200/24 |

## 4.2 VLAN Plan

| VLAN ID | Name | Subnet | Gateway | Devices |
| --- | --- | --- | --- | --- |
| 10 | DEPT_A | 192.168.10.0/24 | 192.168.10.1 | PC0, PC1 |
| 20 | DEPT_B | 192.168.20.0/24 | 192.168.20.1 | PC2, PC3 |
| 30 | DEPT_C | 192.168.30.0/24 | 192.168.30.1 | PC4, PC5 |
| 99 | MANAGEMENT | 192.168.99.0/24 | 192.168.99.1 | All switches |
| 100 | SERVERS | 192.168.100.0/24 | 192.168.100.1 | Server0 |

# 5. VLAN Configuration

VLANs were created on all switches using the following commands (shown for Switch1; repeated on Switch0, Switch2, and Switch3 for their respective VLANs):

Switch1# configure terminal

Switch1(config)# vlan 10

Switch1(config-vlan)# name DEPT_A

Switch1(config-vlan)# vlan 20

Switch1(config-vlan)# name DEPT_B

Switch1(config-vlan)# vlan 30

Switch1(config-vlan)# name DEPT_C

Switch1(config-vlan)# vlan 99

Switch1(config-vlan)# name MANAGEMENT

Switch1(config-vlan)# vlan 100

Switch1(config-vlan)# name SERVERS

Switch1(config-vlan)# exit

# 6. Trunk Configuration

Trunk links carry traffic from multiple VLANs between switches using IEEE 802.1Q encapsulation. VLAN 99 is set as the native VLAN on all trunk ports.

## 6.1 Switch1 Trunk Ports

interface GigabitEthernet0/1

switchport mode trunk

switchport trunk allowed vlan 10,20,30,99,100

switchport trunk native vlan 99

interface FastEthernet0/1

switchport mode trunk

switchport trunk allowed vlan 10,99

switchport trunk native vlan 99

interface FastEthernet0/2

switchport mode trunk

switchport trunk allowed vlan 20,30,99

switchport trunk native vlan 99

## 6.2 Access Switch Trunk Uplinks

! Switch0 uplink to Switch1

interface GigabitEthernet0/1

switchport mode trunk

switchport trunk allowed vlan 10,99

! Switch2 uplink to Switch1

interface FastEthernet0/1

switchport mode trunk

switchport trunk allowed vlan 20,30,99

! Switch3 uplink to Switch2

interface GigabitEthernet0/1

switchport mode trunk

switchport trunk allowed vlan 30,99

# 7. Inter-VLAN Routing

Since all switches are Cisco 2960 (Layer 2 only), inter-VLAN routing is implemented using the Router-on-a-Stick method on Router0. Subinterfaces are created on GigabitEthernet0/0, one per VLAN, each with IEEE 802.1Q encapsulation.

interface GigabitEthernet0/0

no ip address

no shutdown

!

interface GigabitEthernet0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

ip helper-address 192.168.100.200

!

interface GigabitEthernet0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

ip helper-address 192.168.100.200

!

interface GigabitEthernet0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

ip helper-address 192.168.100.200

!

interface GigabitEthernet0/0.99

encapsulation dot1Q 99 native

ip address 192.168.99.1 255.255.255.0

!

interface GigabitEthernet0/0.100

encapsulation dot1Q 100

ip address 192.168.100.1 255.255.255.0

# 8. DHCP Configuration

The DHCP server (Server0 at 192.168.100.200) provides automatic IP addressing for all three department VLANs. The ip helper-address command on each Router0 subinterface relays DHCP broadcasts from clients to the server.

| Pool Name | Start IP | Subnet Mask | Gateway | DNS |
| --- | --- | --- | --- | --- |
| POOL_VLAN10 | 192.168.10.11 | 255.255.255.0 | 192.168.10.1 | .100.200 |
| POOL_VLAN20 | 192.168.20.11 | 255.255.255.0 | 192.168.20.1 | .100.200 |
| POOL_VLAN30 | 192.168.30.11 | 255.255.255.0 | 192.168.30.1 | .100.200 |

# 9. DNS Configuration

The DNS service runs on Server0 alongside DHCP. The following A records were configured through the Packet Tracer Services GUI:

| Hostname | Type | IP Address |
| --- | --- | --- |
| server0.local | A Record | 192.168.100.200 |
| router0.local | A Record | 192.168.10.1 |
| switch1.local | A Record | 192.168.99.1 |

# 10. Verification

## 10.1 show vlan brief — Switch1

Switch1# show vlan brief

VLAN  Name                  Status    Ports

----  --------------------  --------  --------------------

1     default               active

10    DEPT_A                active

20    DEPT_B                active

30    DEPT_C                active

99    MANAGEMENT            active

100   SERVERS               active    Fa0/24

## 10.2 show interfaces trunk — Switch1

Switch1# show interfaces trunk

Port     Mode    Encapsulation  Status     Native vlan

Gi0/1    on      802.1q         trunking   99

Fa0/1    on      802.1q         trunking   99

Fa0/2    on      802.1q         trunking   99

Port     Vlans allowed on trunk

Gi0/1    10,20,30,99,100

Fa0/1    10,99

Fa0/2    20,30,99

## 10.3 Connectivity Test Results

| Test | Source | Destination | Result | Type |
| --- | --- | --- | --- | --- |
| Intra-VLAN ping | PC0 (VLAN 10) | PC1 (VLAN 10) | Pass | Same VLAN |
| Inter-VLAN ping | PC0 (VLAN 10) | PC2 (VLAN 20) | Pass | Via RoaS |
| DHCP lease | PC4 (VLAN 30) | Server0 | Pass | Auto IP |
| DNS resolution | PC0 | server0.local | Pass | DNS |
| Server ping | PC0 | 192.168.100.200 | Pass | Cross-VLAN |

# 11. Conclusion

- A two-router, four-switch enterprise network was successfully designed and implemented in Cisco Packet Tracer

- Five VLANs (10, 20, 30, 99, 100) were created and verified across all switches

- IEEE 802.1Q trunk links established between Switch1 and all connected switches

- Inter-VLAN routing implemented using Router-on-a-Stick with subinterfaces on Router0 (Gi0/0.10, .20, .30, .99, .100)

- DHCP relay configured using ip helper-address on each subinterface; all PCs received IPs automatically

- DNS server configured with A records for internal hostname resolution

- Configuration verified using show vlan brief, show interfaces trunk, and ping tests — all passed