7. VLAN CONFIGURATION, SWITCH CONFIGURATION

AND INTER-VLAN ROUTING

# 1. Objective

To configure VLANs on a switch, assign ports to VLANs, configure switchport modes (access and trunk), and implement Inter-VLAN routing using a router-on-a-stick configuration in Cisco Packet Tracer.

# 2. Topology Overview

The network consists of one router connected to a switch via a trunk link. The switch has four PCs connected to different ports, which are assigned to two VLANs (VLAN 10 and VLAN 20). Inter-VLAN routing is configured on the router using subinterfaces.

Figure 1: Network Topology Diagram

# 3. Devices Used

• 1 × Router (1941)

• 1 × Switch (2960)

• 4 × PC-PT (PC0, PC1, PC2, PC3)

# 4. VLAN and IP Addressing Scheme

The network is segmented into two VLANs:

| VLAN ID | VLAN Name | Network | Gateway |
| --- | --- | --- | --- |
| 10 | Computer | 192.168.10.0/24 | 192.168.10.1 |
| 20 | Electronics | 192.168.20.0/24 | 192.168.20.1 |

## 4.1 Device IP Addressing Table

| Device | Interface/Port | IP Address | VLAN |
| --- | --- | --- | --- |
| Router0 | Gi0/0.10 | 192.168.10.1 | 10 |
| Router0 | Gi0/0.20 | 192.168.20.1 | 20 |
| PC0 | Fa0/1 (Switch) | 192.168.10.2 | 10 |
| PC1 | Fa0/2 (Switch) | 192.168.10.3 | 10 |
| PC2 | Fa0/3 (Switch) | 192.168.20.2 | 20 |
| PC3 | Fa0/4 (Switch) | 192.168.20.3 | 20 |

# 5. Configuration Steps

Figure 2: Configuration Notes

## 5.1 Switch VLAN Configuration

### 5.1.1 Create VLANs

Switch(config)# vlan 10

Switch(config-vlan)# name Computer

Switch(config-vlan)# exit

Switch(config)# vlan 20

Switch(config-vlan)# name Electronics

Switch(config-vlan)# exit

### 5.1.2 Assign Ports to VLANs

For VLAN 10 (Ports Fa0/1-2):

Switch(config)# interface range fa0/1-2

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 10

Switch(config-if-range)# exit

For VLAN 20 (Ports Fa0/3-4):

Switch(config)# interface range fa0/3-4

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 20

Switch(config-if-range)# exit

### 5.1.3 Configure Trunk Port

Switch(config)# interface fa0/5

Switch(config-if)# switchport mode trunk

Switch(config-if)# exit

## 5.2 Router Inter-VLAN Configuration

### 5.2.1 Configure Subinterface for VLAN 10

Router(config)# interface gigabitEthernet 0/0.10

Router(config-subif)# encapsulation dot1Q 10

Router(config-subif)# ip address 192.168.10.1 255.255.255.0

Router(config-subif)# exit

### 5.2.2 Configure Subinterface for VLAN 20

Router(config)# interface gigabitEthernet 0/0.20

Router(config-subif)# encapsulation dot1Q 20

Router(config-subif)# ip address 192.168.20.1 255.255.255.0

Router(config-subif)# exit

### 5.2.3 Enable Physical Interface

Router(config)# interface gigabitEthernet 0/0

Router(config-if)# no shutdown

Router(config-if)# exit

## 5.3 PC IP Configuration

Configure each PC with static IP addresses as shown in the Device IP Addressing Table (Section 4.1), using their respective VLAN gateway addresses.

Subnet mask for all devices: 255.255.255.0

# 6. Verification Commands

## 6.1 Switch Verification

Switch# show vlan brief

Purpose: Displays VLAN configuration and port assignments

## 6.2 Router Verification

Router# show ip interface brief

Purpose: Displays interface status and IP addresses

# 7. Connectivity Testing

## 7.1 Intra-VLAN Testing

From PC0, ping PC1 (both in VLAN 10):

C:\> ping 192.168.10.3

Expected Result: Successful replies (devices in same VLAN communicate directly through switch)

## 7.2 Inter-VLAN Testing

From PC0 (VLAN 10), ping PC2 (VLAN 20):

C:\> ping 192.168.20.2

Expected Result: Successful replies (traffic routed through router subinterfaces)

## 7.3 Gateway Testing

From any PC, ping its default gateway:

From PC0: C:\> ping 192.168.10.1

From PC2: C:\> ping 192.168.20.1

Expected Result: Successful replies

# 8. Observations

• VLANs successfully created and ports assigned to respective VLANs.

• Trunk link established between switch and router for VLAN traffic.

• Devices within the same VLAN can communicate directly through the switch.

• Inter-VLAN communication requires routing through the router subinterfaces.

• Router-on-a-stick configuration successfully enables communication between VLANs.

• All connectivity tests passed without packet loss.

# 9. Result

VLANs were successfully configured on the switch with appropriate port assignments. Inter-VLAN routing was implemented using router subinterfaces with 802.1Q encapsulation. All devices can communicate within their VLANs and across VLANs through the router, demonstrating successful network segmentation and inter-VLAN routing.

# 10. Conclusion

This lab successfully demonstrated VLAN configuration, switchport mode configuration (access and trunk), and Inter-VLAN routing using the router-on-a-stick method. The lab illustrates how VLANs provide network segmentation for security and traffic management, while Inter-VLAN routing enables communication between different VLANs. Understanding these concepts is essential for designing scalable and secure enterprise networks.