WIRELESS NETWORK SETUP AND PACKET ANALYSIS

USING CISCO PACKET TRACER

# 1. Objective

To design and configure a basic wireless local area network (WLAN) using a wireless router and client laptops in Cisco Packet Tracer, and to observe packet flow between connected devices.

# 2. Topology Overview

The network consists of one wireless router (WRT300N) acting as an access point and DHCP server. Two laptops are connected to the router over Wi-Fi.

Figure 1: Network Topology Diagram

# 3. Devices Used

• 1 × WRT300N Wireless Router

• 2 × Laptop-PT (wireless enabled)

# 4. IP Addressing Table

Dynamic IP addressing using DHCP from the wireless router.

| S.N. | Device Name | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- | --- |
| 1 | WRT300N Wireless Router | LAN | 192.168.0.1 | 255.255.255.0 | — |
| 2 | Laptop0 | Wireless0 | DHCP Assigned | 255.255.255.0 | 192.168.0.1 |
| 3 | Laptop1 | Wireless0 | DHCP Assigned | 255.255.255.0 | 192.168.0.1 |

# 5. Configuration Steps

## 5.1 Wireless Router Setup

1. Place WRT300N in the workspace.

2. Open the device → GUI tab.

3. Set SSID (e.g., LabWiFi).

4. Enable DHCP service.

5. Configure wireless security (WPA2-PSK) and set a password.

## 5.2 Laptop Wireless Configuration

For each laptop:

1. Open Desktop → PC Wireless.

2. Connect to the SSID (LabWiFi).

3. Enter the password.

4. IP address will be assigned automatically via DHCP.

# 6. Connectivity Testing

After connection:

1. Go to Desktop → Command Prompt.

2. Use the ping command from Laptop0 to Laptop1.

Example: ping 192.168.0.103

Expected Result: Successful replies confirm proper connectivity.

# 7. Packet Analysis Procedure

1. Switch Packet Tracer to Simulation Mode.

2. Send a ping between the laptops.

3. Observe the following:

• ARP request and reply

• ICMP echo request

• ICMP echo reply

4. Verify the path through the wireless router.

# 8. Observations

• Both laptops successfully received IP addresses from DHCP.

• ARP was used to map IP to MAC address before ICMP transmission.

• Packets traveled wirelessly via the access point.

• Communication was successful without packet loss.

# 9. Result

The wireless LAN was successfully established. End devices communicated with each other, and packet movement was verified using simulation tools.

# 10. Conclusion

This lab demonstrated how to configure a wireless router, connect clients over Wi-Fi, obtain IP addresses automatically, and analyze packet exchange. It improves understanding of WLAN operation, DHCP services, and basic network troubleshooting using Cisco Packet Tracer.